Program and Branch Name:

BTech CSE - F

PBL Progress Report – II

| Field | Details |
| --- | --- |
| Course Name | Computer Architecture & Parallel Processing |
| Course Code | CCSE0304 |
| Faculty Name | Dr. Amrita Bhatnagar |
| Assignment Type | Group Assignment |
| Student(s) Name | Utkarsh Raj Shukla Vivek Kumar Vishal Gupta Yash Srivastava Nishant Kumar Mahto |
| ERP ID (I Year) / Roll No. (II Year onwards) | 2501330100398 2501330100414 2501330100411 2501330100422 0261DCS009 |
| Team/Group No. (Group/Capstone only) | Group No. - 91 |
| Problem Statement / Project Title | Pharmacy Inventory Computing System |
| SDG(s) | SDG 3 – Good Health & Well-Being |
| Reporting Month | Month 2 |
| Date of Submission | 08/10/2026 |

Student Declaration

I/We certify that this assignment is my/our own work, with all sources appropriately acknowledged. I/We understand that a 0% mark and further academic penalties may apply if the content is found to be plagiarised or inappropriately AI-generated.

Student Name & Signature(s):

| S. No. | Student Name | Roll No. | Signature |
| --- | --- | --- | --- |
| 1 | Utkarsh Raj Shukla | 2501330100398 |  |
| 2 | Vivek Kumar | 2501330100414 |  |
| 3 | Vishal Gupta | 2501330100411 |  |
| 4 | Yash Srivastava | 2501330100422 |  |
| 5 | Nishant Kumar Mahto | 0261DCS009 |  |

Date: 08/10/2026

PBL Progress Report – II

Review 2: Development & Application

Review Coverage: Group Assignment — Unit 2 (remaining: division and logic operations, floating point arithmetic, ALU design, IEEE 754) and Unit 3 (Control Unit: instruction types, formats, instruction cycle and sub-cycles, micro-operations, program control, RISC, CISC, pipelining, hardwired and microprogrammed control, horizontal and vertical microprogramming, Flynn’s classification), built on the Unit 1 and Unit 2 study from Review 1.

| Field | Details |
| --- | --- |
| 1. Previous Review Feedback | No feedback has been given by the faculty on Report I yet, so the team moved from concept study to implementation as planned in Report I. |
| 2. Action Taken on Previous Feedback | - Completed every action planned in Report I: requirements, flowchart and design, data structure, Unit 3 study, and implementation. - Went beyond the plan: the system is now a working web app on GitHub Pages, and each CAPP topic is mapped to a feature of it (row 4). |
| 3. Research / Data Collection Progress | - Dataset: 10 sample medicines with batch, units, pack size, reorder level, unit price, expiry, supplier, and Rx/cold-storage flags (assets/js/store.js in the repository). - Study: Dr. Amrita Bhatnagar’s class notes and PPTs for Units 2 and 3; Mano, Computer System Architecture; Stallings, Computer Organization & Architecture; Hamacher, Computer Organization; IEEE Std 754-2019; Booth (1951); Flynn (1966). |
| 4. CAPP Topics Mapped to the Project | CAPP topic; Where in our project; How it is applied. Registers, transfers, addressing (U1); Lab › Instruction cycle; PharmaCPU uses PC, IR, MAR, MBR and R0–R7. It addresses inventory records in memory by immediate, direct, register and base + displacement modes.. ALU design (U2); Stock › Sell, Restock; Lab › ALU; A sale runs SUB stock, sold and a restock runs ADD on an 8-bit ALU with S2 S1 S0 select. A borrow (C = 0) refuses the sale.. Division (U2); Stock › Packs; Lab › Division; Restoring and non-restoring division split loose tablets into full packs and loose tablets: 157 ÷ 10 = 15 packs + 7.. Logic operations (U2); Stock › filters, status; An 8-bit status register per medicine. OR sets the bits; the Reorder and Expiry filters are AND masks 0000 0011 and 0000 1100.. IEEE 754 (U2); Stock › price; Lab › Floating point; Each unit price is stored as a 32-bit word, e.g. ₹6.40 = 0x40CCCCCD (sign, exponent, fraction).. Floating point arithmetic (U2); Stock › value; Lab › Floating point; Stock value = price × units in float32. Add/sub/mul/div are shown as align → operate → normalise → round → pack.. Instruction types & formats (U3); Lab › Instruction set; 16 instructions in four types (data transfer, arithmetic, logical, program control) and five 16-bit formats.. Instruction cycle, micro-ops (U3); Lab › Instruction cycle; Inventory programs run one clock at a time: fetch T0–T2, decode T3, execute T4…, each step a micro-operation such as MAR ← PC.. Program control (U3); Lab › Instruction cycle; JN, JZ and JNZ loops scan every medicine record to count low-stock and expiring items.. RISC vs CISC (U3); Lab › Pipelining; A sale is LOAD, LOAD, SUB, STORE on our RISC machine, compared with one CISC memory-to-memory SUBM.. Pipelining (U3); Lab › Pipelining; A 5-stage timing diagram of the low-stock scan, with RAW stalls, forwarding and branch flushes.. Hardwired vs micro-programmed control (U3); Lab › Control design; Control terms per instruction (e.g. D6·T4 for SUB), and the same steps as 24-bit horizontal / 26-bit vertical microcode words.. Flynn’s classification (U3); Lab › Flynn’s taxonomy; An expiry scan as SISD (one date per step) vs SIMD (4 lanes → 4× fewer steps); MISD and MIMD pharmacy examples.. |
| 5. Methodology / Approach Followed | - Each pharmacy operation was mapped to the CAPP concept that performs it (row 4) and built as a module the user can step through: alu.js (Unit 2), cpu.js (Unit 3), store.js (data), app.js (interface); tested in Node.js, deployed by GitHub Actions. |
| 6. Work Completed During Month 2 | - Stock view: medicine list, search, Reorder/Expiry filters, sell and restock through the ALU, and a detail panel showing the hardware behind each value. - Lab view: 9 topics (ALU, division, multiplication, floating point, instruction set, instruction cycle, control design, pipelining, Flynn), plus automated tests and deployment. |
| 7. Analysis / Development / Implementation | - Memory map: N at 0x100, then records [qty, reorder, status]; four assembly programs run on this data. - Example: selling 30 of 240 runs SUB 240, 30 → 210 (C = 1). Selling 40 of 36 gives C = 0 (a borrow), so the sale is refused. ₹6.40 × 36 = 230.40000915 in float32, bit-exact with the hardware. - Low-stock scan: found 3 medicines (Amoxicillin, Metformin, ORS), the same as the Stock view, in 85 instructions and 535 clock cycles. |
| 8. Results / Interim Outcomes | Program (sample data); Instructions; Non-pipelined; Pipelined, no fwd; Pipelined + fwd. Low-stock count; 85; 425 cycles; 147 (2.89×); 137 (3.10×). Total units in stock; 55; 275 cycles; 97 (2.84×); 87 (3.16×). Expiry alert scan; 71; 355 cycles; 123 (2.89×); 113 (3.14×). - FP results matched the hardware FPU in 80,000/80,000 random tests; all 84,374 automated checks pass. |
| 9. Evidence / Supporting Material | - Repository: github.com/utk042/CAPP-PBL · Live app: utk042.github.io/CAPP-PBL/ - Class notes and PPTs (Units 2 and 3) · Reference textbooks · Report I |
| 10. Challenges & Corrective Actions | - Non-restoring division failed for large divisors → A register widened to n + 2 bits. FP rounding errors → guard/round/sticky bits, checked against hardware. A dense first UI → redesigned as a simple list plus one Lab topic at a time. |
| 11. Individual / Team Contribution | Member; Roll No.; Contribution. Utkarsh Raj Shukla; 2501330100398; PharmaCPU ISA, assembler, instruction cycle, integration, GitHub. Vivek Kumar; 2501330100414; ALU design, IEEE 754 encoding, floating point arithmetic. Vishal Gupta; 2501330100411; Division, Booth’s algorithm, logic operations, status register. Yash Srivastava; 2501330100422; Pipelining, RISC vs CISC, Flynn’s taxonomy, testing. Nishant Kumar Mahto; 0261DCS009; Stock view, control-design view, report compilation. |
| 12. Overall Progress (%) | 60% |
| 13. Plan for Final Review | - Unit 4: cache simulator (direct and set-associative mapping) for medicine records, virtual memory, and MSI/MESI/Dragon coherence for several billing counters. - Unit 5: directory protocols, memory consistency, LL-SC and barrier synchronisation for concurrent sales, and interconnects for multi-branch stores; final report and demo. |

Faculty Evaluation:

- To be completed by Faculty in Moodle