Program and Branch Name:

[B.Tech CSE-F]

PBL Progress Report – II

| Field | Details |
| --- | --- |
| Course Name | Computer Architecture & Parallel Processing |
| Course Code | CCSE0304 |
| Faculty Name | Dr. Amrita Bhatnagar |
| Assignment Type | Group Assignment |
| Student(s) Name | Utkarsh Raj Shukla |
| ERP ID (I Year) / Roll No. (II Year onwards) | 0251CSE180 / 2501330100398 |
| Team/Group No. (Group/Capstone only) | 91 |
| Problem Statement / Project Title | Pharmacy Inventory Computing System |
| SDG(s) | SDG 3 — Good Health & Well - Being |
| Reporting Month | Month 2 |
| Date of Submission | 08/10/2026 |

Student Declaration

I certify that this assignment is my own work, with all sources appropriately acknowledged. I understand that a 0% mark and further academic penalties may apply if the content is found to be plagiarised or inappropriately AI-generated.

Student Name & Signature: Utkarsh Raj Shukla  &

Date: 08/10/2026

PBL Progress Report – II

Review 2: Development & Application

Review Coverage:

- Group Assignment: Unit/Module 2 (Division to IEEE 754) and Unit/Module 3 (Control Unit)

| Field | Details |
| --- | --- |
| 1. Previous Review Feedback | No feedback has been given by the faculty on Progress Report – I yet. Work therefore continued as planned in the “Plan for Next Review” section of Review 1. |
| 2. Action Taken on Previous Feedback | No feedback corrections were pending. All five actions planned in Review 1 have been completed: - Finalised the system requirements and features. - Prepared the system flowchart, data structure and basic design. - Built the database/system structure: medicine records, an 8-bit status register and a memory map. - Applied the remaining Unit 2 and all Unit 3 concepts to the project. - Implemented the system as a working web application and hosted it on GitHub Pages. |
| 3. Research / Data Collection Progress | - Studied class notes and lecture PPTs for Unit 2 (division, logic operations, floating point arithmetic, ALU design, IEEE 754) and Unit 3 (instruction formats, instruction cycle, micro-operations, RISC/CISC, pipelining, hardwired and microprogrammed control, Flynn’s classification). - Referred to M. Morris Mano (Computer System Architecture), W. Stallings (Computer Organization & Architecture), C. Hamacher (Computer Organization) and the IEEE 754 standard. - Collected sample pharmacy data for 10 medicines: category, batch, units, pack size, reorder level, unit price, expiry date, supplier, and prescription/cold-storage status. - Studied web technologies (HTML5, CSS3, JavaScript, browser localStorage) and GitHub Pages deployment. |
| 4. Unit/Module Concepts Applied | - Division (Unit 2): restoring and non-restoring division convert loose tablets into packs/strips (quotient) and loose tablets (remainder). - Logic operations (Unit 2): each medicine has an 8-bit status register (Low, Out, Expiring, Expired, Rx, Cold, High value). Selective set (OR), mask test (AND), selective clear (AND with M′) and selective complement (XOR) drive the alerts and the inventory filter. - ALU design (Unit 2): an 8-bit ALU with a 3-bit function select (ADD, SUB, AND, OR, XOR, NOT, SHL, SHR) and C/Z/N/V flags. Each sale is an ALU SUB and each purchase an ALU ADD. A borrow (C = 0) blocks overselling. - Floating point & IEEE 754 (Unit 2): medicine prices are shown as single- and double-precision sign/exponent/fraction bits. Stock value is computed in single precision, and FP add/sub/mul/div are shown step by step (align, operate, normalise, round to nearest even, pack). - Instruction types, formats & cycle (Unit 3): “PharmaCPU” has 16-bit words, 16 instructions and 5 formats (R, I, M, X, B), covering data transfer, arithmetic, logical and program-control types. Fetch, decode and execute sub-cycles run clock by clock as micro-operations (RTL). - Program control, RISC & CISC (Unit 3): JMP/JZ/JN/JNZ loops scan the inventory records. A RISC load/store sequence is compared with a CISC memory-to-memory instruction. - Pipelining (Unit 3): a 5-stage IF–ID–EX–MEM–WB timing diagram shows RAW data hazards, operand forwarding, branch flushes, CPI and speed-up. - Hardwired & microprogrammed control (Unit 3): the hardwired view shows the T-states and the opcode decoder (Dᵢ·Tⱼ). The microprogrammed view shows the control memory, the CAR and the mapping logic. Horizontal microcode is 24 bits (one bit per signal) and vertical microcode is 26 bits in encoded fields F1/F2/F3, CD, BR and AD. - Flynn’s classification (Unit 3): an animated SISD vs SIMD expiry scan, plus MISD and MIMD pharmacy examples. |
| 5. Methodology / Approach Followed | - Iterative development: requirements → design → module-wise implementation → testing → deployment. - Each pharmacy computation was mapped to the hardware concept that performs it, and that concept is shown step by step in the app. - Modular design: alu.js (Unit 2), cpu.js (Unit 3), store.js (inventory data) and app.js (user interface). - Verification: an automated test script (tests/run.js) checks the ALU, Booth, division, IEEE 754 arithmetic (compared with the hardware FPU) and the CPU programs. - Version control with Git/GitHub. GitHub Actions runs the tests and deploys the site to GitHub Pages on every push. |
| 6. Work Completed During Month 2 | - Finalised the requirements and designed the medicine record, the status register and the CPU memory map (records at 0x100). - Built the inventory module: add, edit and delete medicines, sales and purchases through the ALU, search, bit-mask filter, JSON import/export, and dashboard alerts and charts. - Built the ALU Lab: ALU design, division, Booth multiplication, logic operations, IEEE 754 and FP arithmetic. - Built the Control Unit Lab: ISA and formats, assembler, instruction-cycle simulator, hardwired and microprogrammed control, pipelining, RISC vs CISC, and Flynn’s classification. - Tested the system and deployed it as a web app on GitHub Pages. Prepared this progress report. |
| 7. Analysis / Development / Implementation | The system is a web application written in HTML5, CSS3 and JavaScript with no external frameworks. Data is stored in the browser (localStorage). - Medicine record: name, category, batch, units, pack size, reorder level, unit price, expiry, supplier, and Rx/cold flags. The status register is computed automatically. - Memory map for PharmaCPU: item count N at 0x100, then 3-word records [qty, reorder, status], with results at 0x1F0–0x1F2. - Assembly programs written and run: (i) count low-stock medicines, (ii) total units in stock, (iii) expiry alert scan using an AND mask, (iv) record a sale (load, compute, store). - The web app is responsive and works on mobile phones and desktops, in light and dark themes. |
| 8. Results / Interim Outcomes | - A working web app is live at https://utk042.github.io/CAPP-PBL/. - With the sample data, the system raised 3 low/out-of-stock alerts and 5 expired/expiring alerts automatically. - The PharmaCPU low-stock program counted the same 3 medicines as the app. It ran 85 instructions in 535 clock cycles (T-states). - Pipelining the same program reduced 425 cycles (non-pipelined) to 137 cycles with forwarding (CPI 1.61, 3.10× speed-up) and to 147 cycles without forwarding (2.89× speed-up). - Floating point results matched the hardware FPU in all 80,000 random test cases. All 84,374 automated checks pass. |
| 9. Evidence / Supporting Material | - GitHub repository (source code): https://github.com/utk042/CAPP-PBL - Live web app (GitHub Pages): https://utk042.github.io/CAPP-PBL/ - Class notes and lecture PPTs of Unit 2 (division, IEEE 754, ALU design) and Unit 3 (instruction cycle, micro-operations, control unit, pipelining, Flynn’s classification) were used to design each module. - Reference textbooks: Mano, Stallings and Hamacher. IEEE 754 standard. - Screenshots of the dashboard, inventory, ALU Lab and Control Unit Lab, and the automated test output (tests/run.js). - PBL Progress Report – I (Month 1), with the requirement list and initial design. |
| 10. Challenges & Corrective Actions | - Non-restoring division gave wrong results for large divisors (accumulator overflow). The A register was widened to n + 2 bits, and all 8-bit cases were verified. - Booth’s algorithm overflows for −128. The input range was limited to −127…127, and every product in that range was verified. - Correct floating point rounding. Exact significands with guard, round and sticky bits were used, and the results were compared with the hardware FPU. - Showing pipeline hazards clearly. Fixed rules were adopted (write in the first half of WB, read in the second half of ID, branch resolved in EX), and stalls are marked in the diagram. - Making detailed simulators usable on small screens. A responsive layout with scrollable tables was built. |
| 11. Individual / Team Contribution | - Designed the system structure, data model and memory map. - Implemented the inventory module, the ALU Lab and the Control Unit Lab of the web app. - Wrote the automated tests, set up GitHub version control and deployed the app on GitHub Pages. - Prepared the project documentation (README) and this progress report. |
| 12. Overall Progress (%) | 50% – Problem identification, research and Unit 1 & 2 study (Month 1), plus the design, implementation, testing and deployment of the web app covering the remaining Unit 2 and all of Unit 3 (Month 2), are complete. |
| 13. Plan for Final Review | - Unit 4: add a cache-memory simulator (direct-mapped and set-associative) for frequently accessed medicine records, and cover virtual memory. - Unit 4: simulate the snooping coherence protocols MSI, MESI and Dragon, and directory-based coherence, for several billing counters sharing stock data. - Unit 5: memory consistency models and synchronisation (LL-SC, barriers) for concurrent sales; interconnect topologies for a multi-branch pharmacy network. - Final testing, user guide, final report and live demonstration. |

Faculty Evaluation:

- To be completed by Faculty in Moodle

Conclusion

During Month 2, the Pharmacy Inventory Computing System was developed into a working web application, hosted on GitHub Pages. It handles medicine stock, sales and purchases, expiry and low-stock alerts, and carries out each computation with the remaining Unit 2 and all Unit 3 concepts: division, logic operations, ALU design, IEEE 754 floating point, instruction formats and cycle, micro-operations, hardwired and microprogrammed control, pipelining, RISC/CISC and Flynn’s classification. The project is now 50% complete. Units 4 and 5 (cache, coherence protocols and parallel systems) will be added for the final review.